ELE504

Electronic Circuits II
F2024

Dr. Rueda

MP

ELE504 MP Lab Report 1

October 2nd, 2024

October 3rd, 2024

Shai Mourad/Jason Fu

501170415/501035603

SM/JF

Table of Content

1. Introduction...........................................................................................R2

2. Circuit under Test..................................................................................R2

3. Experimental Results............................................................................R3

4. Conclusions and Remarks.....................................................................R4

5. Appendix…………………………………………...............................R4

### Introduction

For this lab experiment, the first portion of the major project was completed. An inverting integrator was placed in series with a non-inverting bistable multivibrator to create a waveform generator. Using the design for a waveform generator from the pre-lab for a specific fx value, the circuit was implemented on a breadboard to output a square and triangular waveform with proper output values. This lab took place on October 2nd, 2024.

### Circuit under Test

Waveform Generator Design For fx = 3100Hz

### Experimental Results

Figure 1. Waveforms of both VO1 and VO2

| Component | Pre-Lab Value | In-Lab Value |
| --- | --- | --- |
| C | 0.01μF | 0.01μF |
| R | 20kΩ | 20kΩ |
| R1 | 10kΩ | 10kΩ |
| R2 | 26kΩ | 26kΩ |
| L± | ±12V | ±12V |

Table 1. Values of Components Used in Pre-Lab and In-Lab

| Square Waveform (VO1) | Pre-Lab | Simulation | In-Lab |
| --- | --- | --- | --- |
| VTH | 10.5V | 10.647V | 11.3V |
| VTL | -10.5V | -10.287V | -11.3V |
| fx | 3100Hz | 3154.57Hz | 3152Hz |

Table 2a. Comparing Pre-Lab, Multisim Simulation, and Experimental Values for VO1

| Triangular Waveform (VO2) | Pre-Lab | Simulation | In-Lab |
| --- | --- | --- | --- |
| VTH | 4V | 3.543V | 5.17V |
| VTL | -4V | -3.650V | -5.17V |
| fx | 3100Hz | 3144.65Hz | 3152Hz |

Table 2b. Comparing Pre-Lab and Multisim Simulation, and Experimental Values for VO2

### Conclusion and Remarks

### The result of the in-lab experiment successfully demonstrated the waveforms and produced both square and triangular waveforms, the same as the prelab analysis and simulation. The experimental results closely aligned with the theoretical value and the simulation results. Overall, the experiment validated the design and implementation of the circuit, confirming its effectiveness for generating two waveforms.

### Appendix

References

Kassam, M. S. (2022). Major Project - MP [PDF]. D2L. https://courses.torontomu.ca/d2l/le/content/913319/viewContent/5864651/View